OBDII Code Info

|  | If your diagnostic connector looks like this one,then you have an OBDII sytem,and it requires a scanner to access the codes.Scanners can be found from the links on the homepage.All vehicles after 96 have the OBDII systems and some 95 models,too. |
| --- | --- |

| OBDII Code Definitions |  |  |
| --- | --- | --- |
| Pin 2 - J1850 Bus+ Pin 4 - Chassis Ground Pin 5 - Signal Ground Pin 6 - CAN High (J-2284) Pin 7 - ISO 9141-2 K Line Pin 10 - J1850 Bus Pin 14 - CAN Low (J-2284) Pin 15 - ISO 9141-2 L Line Pin 16 - Battery Power | Some 1995 GM vehicles have the 16-pin connector but have accessible OBD-I codes. Check the underhood emissions sticker to confirm, and if yours is OBD-I, the codes are accessed by bridging terminals 5 and 6. If your vehicle is OBD-II, a scan tool or computer is required to access the codes. |  |

P 0 1 0 1 
1  2 3 4 5

1st digit
P = powertrain
B = Body
C = Chassis

2nd digit
0 = Standard
1 = Manufacturer specific

3rd digit
1 = Emission management
2 = Injector circuit
3 = Ignition 
4 = Auxiliary emission
5 = Vehicle speed & idle control
6 = Computer & output circuit
7 = Transmission

# OBDII P Code Definitions

| P0100 | Mass or Volume Air Flow Circuit Malfunction |
| --- | --- |
| P0101 | Mass or Volume Air Flow Circuit Range/Performance Problem |
| P0102 | Mass or Volume Air Flow Circuit Low Input |
| P0103 | Mass or Volume Air Flow Circuit High Input |
| P0104 | Mass or Volume Air Flow Circuit Intermittent |
| P0105 | Manifold Absolute Pressure/Barometric Pressure Circuit Malfunction |
| P0106 | Manifold Absolute Pressure/Barometric Pressure Circuit Range/Performance Problem |
| P0107 | Manifold Absolute Pressure/Barometric Pressure Circuit Low Input |
| P0108 | Manifold Absolute Pressure/Barometric Pressure Circuit High Input |
| P0109 | Manifold Absolute Pressure/Barometric Pressure Circuit Intermittent |
| P0109 | Intake Air Temperature Circuit Malfunction |
| P0110 | IAT Circuit Malfunction |
| P0111 | Intake Air Temperature Circuit Range/Performance Problem |
| P0112 | Intake Air Temperature Circuit Low Input |
| P0113 | Intake Air Temperature Circuit High Input |
| P0114 | Intake Air Temperature Circuit Intermittent |
| P0115 | Engine Coolant Temperature Circuit Malfunction |
| P0116 | Engine Coolant Temperature Circuit Range/Performance Problem |
| P0117 | Engine Coolant Temperature Circuit Low Input |
| P0118 | Engine Coolant Temperature Circuit High Input |
| P0119 | Engine Coolant Temperature Circuit Intermittent |
| P0120 | Throttle Position Sensor/Switch A Circuit Malfunction |
| P0121 | Throttle Position Sensor/Switch A Circuit Range/Performance Problem |
| P0122 | Throttle Position Sensor/Switch A Circuit Low Input |
| P0123 | Throttle Position Sensor/Switch A Circuit High Input |
| P0124 | Throttle Position Sensor/Switch A Circuit Intermittent |
| P0125 | Insufficient Coolant Temperature for Closed Loop Fuel Control |
| P0126 | Insufficient Coolant Temperature for Stable Operation |
| P0128 | Coolant Thermostat Malfunction |
| P0130 | O2 Sensor Circuit Malfunction (Bank 1 Sensor 1) |
| P0131 | O2 Sensor Circuit Low Voltage (Bank 1 Sensor 1) |
| P0132 | O2 Sensor Circuit High Voltage (Bank 1 Sensor 1) |
| P0133 | O2 Sensor Circuit Slow Response (Bank 1 Sensor 1) |
| P0134 | O2 Sensor Circuit No Activity Detected (Bank 1 Sensor 1) |
| P0135 | O2 Sensor Heater Circuit Malfunction (Bank 1 Sensor 1) |
| P0136 | O2 Sensor Circuit Malfunction (Bank 1 Sensor 2) |
| P0137 | O2 Sensor Circuit Low Voltage (Bank 1 Sensor 2) |
| P0138 | O2 Sensor Circuit High Voltage (Bank 1 Sensor 2) |
| P0139 | O2 Sensor Circuit Slow Response (Bank 1 Sensor 2) |
| P0140 | O2 Sensor Circuit No Activity Detected (Bank 1 Sensor 2) |
| P0141 | O2 Sensor Heater Circuit Malfunction (Bank 1 Sensor 2) |
| P0142 | O2 Sensor Circuit Malfunction (Bank 1 Sensor 3) |
| P0143 | O2 Sensor Circuit Low Voltage (Bank 1 Sensor 3) |
| P0144 | O2 Sensor Circuit High Voltage (Bank 1 Sensor 3) |
| P0145 | O2 Sensor Circuit Slow Response (Bank 1 Sensor 3) |
| P0146 | O2 Sensor Circuit No Activity Detected (Bank 1 Sensor 3) |
| P0147 | O2 Sensor Heater Circuit Malfunction (Bank 1 Sensor 3) |
| P0150 | O2 Sensor Circuit Malfunction (Bank 2 Sensor 1) |
| P0151 | O2 Sensor Circuit Low Voltage (Bank 2 Sensor 1) |
| P0152 | O2 Sensor Circuit High Voltage (Bank 2 Sensor 1) |
| P0153 | O2 Sensor Circuit Slow Response (Bank 2 Sensor 1) |
| P0154 | O2 Sensor Circuit No Activity Detected (Bank 2 Sensor 1) |
| P0155 | O2 Sensor Heater Circuit Malfunction (Bank 2 Sensor 1) |
| P0156 | O2 Sensor Circuit Malfunction (Bank 2 Sensor 2) |
| P0157 | O2 Sensor Circuit Low Voltage (Bank 2 Sensor 2) |
| P0158 | O2 Sensor Circuit High Voltage (Bank 2 Sensor 2) |
| P0159 | O2 Sensor Circuit Slow Response (Bank 2 Sensor 2) |
| P0160 | O2 Sensor Circuit No Activity Detected (Bank 2 Sensor 2) |
| P0161 | O2 Sensor Heater Circuit Malfunction (Bank 2 Sensor 2) |
| P0162 | O2 Sensor Circuit Malfunction (Bank 2 Sensor 3) |
| P0163 | O2 Sensor Circuit Low Voltage (Bank 2 Sensor 3) |
| P0164 | O2 Sensor Circuit High Voltage (Bank 2 Sensor 3) |
| P0165 | O2 Sensor Circuit Slow Response (Bank 2 Sensor 3) |
| P0166 | O2 Sensor Circuit No Activity Detected (Bank 2 Sensor 3) |
| P0167 | O2 Sensor Heater Circuit Malfunction (Bank 2 Sensor 3) |
| P0170 | Fuel Trim Malfunction (Bank 1) |
| P0171 | System too Lean (Bank 1) |
| P0172 | System too Rich (Bank 1) |
| P0173 | Fuel Trim Malfunction (Bank 2) |
| P0174 | System too Lean (Bank 2) |
| P0175 | System too Rich (Bank 2) |
| P0176 | Fuel Composition Sensor Circuit Malfunction |
| P0177 | Fuel Composition Sensor Circuit Range/Performance |
| P0178 | Fuel Composition Sensor Circuit Low Input |
| P0179 | Fuel Composition Sensor Circuit High Input |
| P0180 | Fuel Temperature Sensor A Circuit Malfunction |
| P0181 | Fuel Temperature Sensor A Circuit Range/Performance |
| P0182 | Fuel Temperature Sensor A Circuit Low Input |
| P0183 | Fuel Temperature Sensor A Circuit High Input |
| P0184 | Fuel Temperature Sensor A Circuit Intermittent |
| P0185 | Fuel Temperature Sensor B Circuit Malfunction |
| P0186 | Fuel Temperature Sensor B Circuit Range/Performance |
| P0187 | Fuel Temperature Sensor B Circuit Low Input |
| P0188 | Fuel Temperature Sensor B Circuit High Input |
| P0189 | Fuel Temperature Sensor B Circuit Intermittent |
| P0190 | Fuel Rail Pressure Sensor Circuit Malfunction |
| P0191 | Fuel Rail Pressure Sensor Circuit Range/Performance |
| P0192 | Fuel Rail Pressure Sensor Circuit Low Input |
| P0193 | Fuel Rail Pressure Sensor Circuit High Input |
| P0194 | Fuel Rail Pressure Sensor Circuit Intermittent |
| P0195 | Engine Oil Temperature Sensor Malfunction |
| P0196 | Engine Oil Temperature Sensor Range/Performance |
| P0197 | Engine Oil Temperature Sensor Low |
| P0198 | Engine Oil Temperature Sensor High |
| P0199 | Engine Oil Temperature Sensor Intermittent |
| P0200 | Injector Circuit Malfunction |
| P0201 | Injector Circuit Malfunction - Cylinder 1 |
| P0202 | Injector Circuit Malfunction - Cylinder 2 |
| P0203 | Injector Circuit Malfunction - Cylinder 3 |
| P0204 | Injector Circuit Malfunction - Cylinder 4 |
| P0205 | Injector Circuit Malfunction - Cylinder 5 |
| P0206 | Injector Circuit Malfunction - Cylinder 6 |
| P0207 | Injector Circuit Malfunction - Cylinder 7 |
| P0208 | Injector Circuit Malfunction - Cylinder 8 |
| P0209 | Injector Circuit Malfunction - Cylinder 9 |
| P0210 | Injector Circuit Malfunction - Cylinder 10 |
| P0211 | Injector Circuit Malfunction - Cylinder 11 |
| P0212 | Injector Circuit Malfunction - Cylinder 12 |
| P0213 | Cold Start Injector 1 Malfunction |
| P0214 | Cold Start Injector 2 Malfunction |
| P0215 | Engine Shutoff Solenoid Malfunction |
| P0216 | Injection Timing Control Circuit Malfunction |
| P0217 | Engine Overtemp Condition |
| P0218 | Transmission Over Temperature Condition |
| P0219 | Engine Overspeed Condition |
| P0220 | Throttle/Petal Position Sensor/Switch B Circuit Malfunction |
| P0221 | Throttle/Petal Position Sensor/Switch B Circuit Range/Performance Problem |
| P0222 | Throttle/Petal Position Sensor/Switch B Circuit Low Input |
| P0223 | Throttle/Petal Position Sensor/Switch B Circuit High Input |
| P0224 | Throttle/Petal Position Sensor/Switch B Circuit Intermittent |
| P0225 | Throttle/Petal Position Sensor/Switch C Circuit Malfunction |
| P0226 | Throttle/Petal Position Sensor/Switch C Circuit Range/Performance Problem |
| P0227 | Throttle/Petal Position Sensor/Switch C Circuit Low Input |
| P0228 | Throttle/Petal Position Sensor/Switch C Circuit High Input |
| P0229 | Throttle/Petal Position Sensor/Switch C Circuit Intermittent |
| P0230 | Fuel Pump Primary Circuit Malfunction |
| P0231 | Fuel Pump Secondary Circuit Low |
| P0232 | Fuel Pump Secondary Circuit High |
| P0233 | Fuel Pump Secondary Circuit Intermittent |
| P0234 | Engine Overboost Condition |
| P0235 | Turbocharger Boost Sensor A Circuit Malfunction |
| P0236 | Turbocharger Boost Sensor A Circuit Range/Performance |
| P0237 | Turbocharger Boost Sensor A Circuit Low |
| P0238 | Turbocharger Boost Sensor A Circuit High |
| P0239 | Turbocharger Boost Sensor B Malfunction |
| P0240 | Turbocharger Boost Sensor B Circuit Range/Performance |
| P0241 | Turbocharger Boost Sensor B Circuit Low |
| P0242 | Turbocharger Boost Sensor B Circuit High |
| P0243 | Turbocharger Wastegate Solenoid A Malfunction |
| P0244 | Turbocharger Wastegate Solenoid A Range/Performance |
| P0245 | Turbocharger Wastegate Solenoid A Low |
| P0246 | Turbocharger Wastegate Solenoid A High |
| P0247 | Turbocharger Wastegate Solenoid B Malfunction |
| P0248 | Turbocharger Wastegate Solenoid B Range/Performance |
| P0249 | Turbocharger Wastegate Solenoid B Low |
| P0250 | Turbocharger Wastegate Solenoid B High |
| P0251 | Injection Pump Fuel Metering Control "A" Malfunction (Cam/Rotor/Injector) |
| P0252 | Injection Pump Fuel Metering Control "A" Range/Performance (Cam/Rotor/Injector) |
| P0253 | Injection Pump Fuel Metering Control "A" Low (Cam/Rotor/Injector) |
| P0254 | Injection Pump Fuel Metering Control "A" High (Cam/Rotor/Injector) |
| P0255 | Injection Pump Fuel Metering Control "A" Intermittent (Cam/Rotor/Injector) |
| P0256 | Injection Pump Fuel Metering Control "B" Malfunction (Cam/Rotor/Injector) |
| P0257 | Injection Pump Fuel Metering Control "B" Range/Performance (Cam/Rotor/Injector) |
| P0258 | Injection Pump Fuel Metering Control "B" Low (Cam/Rotor/Injector) |
| P0259 | Injection Pump Fuel Metering Control "B" High (Cam/Rotor/Injector) |
| P0260 | Injection Pump Fuel Metering Control "B" Intermittent (Cam/Rotor/Injector) |
| P0261 | Cylinder 1 Injector Circuit Low |
| P0262 | Cylinder 1 Injector Circuit High |
| P0263 | Cylinder 1 Contribution/Balance Fault |
| P0264 | Cylinder 2 Injector Circuit Low |
| P0265 | Cylinder 2 Injector Circuit High |
| P0266 | Cylinder 2 Contribution/Balance Fault |
| P0267 | Cylinder 3 Injector Circuit Low |
| P0268 | Cylinder 3 Injector Circuit High |
| P0269 | Cylinder 3 Contribution/Balance Fault |
| P0270 | Cylinder 4 Injector Circuit Low |
| P0271 | Cylinder 4 Injector Circuit High |
| P0272 | Cylinder 4 Contribution/Balance Fault |
| P0273 | Cylinder 5 Injector Circuit Low |
| P0274 | Cylinder 5 Injector Circuit High |
| P0275 | Cylinder 5 Contribution/Balance Fault |
| P0276 | Cylinder 6 Injector Circuit Low |
| P0277 | Cylinder 6 Injector Circuit High |
| P0278 | Cylinder 6 Contribution/Balance Fault |
| P0279 | Cylinder 7 Injector Circuit Low |
| P0280 | Cylinder 7 Injector Circuit High |
| P0281 | Cylinder 7 Contribution/Balance Fault |
| P0282 | Cylinder 8 Injector Circuit Low |
| P0283 | Cylinder 8 Injector Circuit High |
| P0284 | Cylinder 8 Contribution/Balance Fault |
| P0285 | Cylinder 9 Injector Circuit Low |
| P0286 | Cylinder 9 Injector Circuit High |
| P0287 | Cylinder 9 Contribution/Balance Fault |
| P0288 | Cylinder 10 Injector Circuit Low |
| P0289 | Cylinder 10 Injector Circuit High |
| P0290 | Cylinder 10 Contribution/Balance Fault |
| P0291 | Cylinder 11 Injector Circuit Low |
| P0292 | Cylinder 11 Injector Circuit High |
| P0293 | Cylinder 11 Contribution/Balance Fault |
| P0294 | Cylinder 12 Injector Circuit Low |
| P0295 | Cylinder 12 Injector Circuit High |
| P0296 | Cylinder 12 Contribution/Range Fault |
| P0300 | Random/Multiple Cylinder Misfire Detected |
| P0301 | Cylinder 1 Misfire Detected |
| P0302 | Cylinder 2 Misfire Detected |
| P0303 | Cylinder 3 Misfire Detected |
| P0304 | Cylinder 4 Misfire Detected |
| P0305 | Cylinder 5 Misfire Detected |
| P0306 | Cylinder 6 Misfire Detected |
| P0307 | Cylinder 7 Misfire Detected |
| P0308 | Cylinder 8 Misfire Detected |
| P0309 | Cylinder 9 Misfire Detected |
| P0310 | Cylinder 10 Misfire Detected |
| P0311 | Cylinder 11 Misfire Detected |
| P0312 | Cylinder 12 Misfire Detected |
| P0320 | Ignition/Distributor Engine Speed Input Circuit Malfunction |
| P0321 | Ignition/Distributor Engine Speed Input Circuit Range/Performance |
| P0322 | Ignition/Distributor Engine Speed Input Circuit No Signal |
| P0323 | Ignition/Distributor Engine Speed Input Circuit Intermittent |
| P0325 | Knock Sensor 1 Circuit Malfunction (Bank 1 or Single Sensor) |
| P0326 | Knock Sensor 1 Circuit Range/Performance (Bank 1 or Single Sensor) |
| P0327 | Knock Sensor 1 Circuit Low Input (Bank 1 or Single Sensor) |
| P0328 | Knock Sensor 1 Circuit High Input (Bank 1 or Single Sensor) |
| P0329 | Knock Sensor 1 Circuit Intermittent (Bank 1 or Single Sensor) |
| P0330 | Knock Sensor 2 Circuit Malfunction (Bank 2) |
| P0331 | Knock Sensor 2 Circuit Range/Performance (Bank 2) |
| P0332 | Knock Sensor 2 Circuit Low Input (Bank 2) |
| P0333 | Knock Sensor 2 Circuit High Input (Bank 2) |
| P0334 | Knock Sensor 2 Circuit Intermittent (Bank 2) |
| P0335 | Crankshaft Position Sensor A Circuit Malfunction |
| P0336 | Crankshaft Position Sensor A Circuit Range/Performance |
| P0337 | Crankshaft Position Sensor A Circuit Low Input |
| P0338 | Crankshaft Position Sensor A Circuit High Input |
| P0339 | Crankshaft Position Sensor A Circuit Intermittent |
| P0340 | Camshaft Position Sensor Circuit Malfunction |
| P0341 | Camshaft Position Sensor Circuit Range/Performance |
| P0342 | Camshaft Position Sensor Circuit Low Input |
| P0343 | Camshaft Position Sensor Circuit High Input |
| P0344 | Camshaft Position Sensor Circuit Intermittent |
| P0350 | Ignition Coil Primary/Secondary Circuit Malfunction |
| P0351 | Ignition Coil A Primary/Secondary Circuit Malfunction |
| P0352 | Ignition Coil B Primary/Secondary Circuit Malfunction |
| P0353 | Ignition Coil C Primary/Secondary Circuit Malfunction |
| P0354 | Ignition Coil D Primary/Secondary Circuit Malfunction |
| P0355 | Ignition Coil E Primary/Secondary Circuit Malfunction |
| P0356 | Ignition Coil F Primary/Secondary Circuit Malfunction |
| P0357 | Ignition Coil G Primary/Secondary Circuit Malfunction |
| P0358 | Ignition Coil H Primary/Secondary Circuit Malfunction |
| P0359 | Ignition Coil I Primary/Secondary Circuit Malfunction |
| P0360 | Ignition Coil J Primary/Secondary Circuit Malfunction |
| P0361 | Ignition Coil K Primary/Secondary Circuit Malfunction |
| P0362 | Ignition Coil L Primary/Secondary Circuit Malfunction |
| P0370 | Timing Reference High Resolution Signal A Malfunction |
| P0371 | Timing Reference High Resolution Signal A Too Many Pulses |
| P0372 | Timing Reference High Resolution Signal A Too Few Pulses |
| P0373 | Timing Reference High Resolution Signal A Intermittent/Erratic Pulses |
| P0374 | Timing Reference High Resolution Signal A No Pulses |
| P0375 | Timing Reference High Resolution Signal B Malfunction |
| P0376 | Timing Reference High Resolution Signal B Too Many Pulses |
| P0377 | Timing Reference High Resolution Signal B Too Few Pulses |
| P0378 | Timing Reference High Resolution Signal B Intermittent/Erratic Pulses |
| P0379 | Timing Reference High Resolution Signal B No Pulses |
| P0380 | Glow Plug/Heater Circuit "A" Malfunction |
| P0381 | Glow Plug/Heater Indicator Circuit Malfunction |
| P0382 | Exhaust Gas Recirculation Flow Malfunction |
| P0385 | Crankshaft Position Sensor B Circuit Malfunction |
| P0386 | Crankshaft Position Sensor B Circuit Range/Performance |
| P0387 | Crankshaft Position Sensor B Circuit Low Input |
| P0388 | Crankshaft Position Sensor B Circuit High Input |
| P0389 | Crankshaft Position Sensor B Circuit Intermittent |
| P0400 | Exhaust Gas Recirculation Flow Malfunction |
| P0401 | Exhaust Gas Recirculation Flow Insufficient Detected |
| P0402 | Exhaust Gas Recirculation Flow Excessive Detected |
| P0403 | Exhaust Gas Recirculation Circuit Malfunction |
| P0404 | Exhaust Gas Recirculation Circuit Range/Performance |
| P0405 | Exhaust Gas Recirculation Sensor A Circuit Low |
| P0406 | Exhaust Gas Recirculation Sensor A Circuit High |
| P0407 | Exhaust Gas Recirculation Sensor B Circuit Low |
| P0408 | Exhaust Gas Recirculation Sensor B Circuit High |
| P0410 | Secondary Air Injection System Malfunction |
| P0411 | Secondary Air Injection System Incorrect Flow Detected |
| P0412 | Secondary Air Injection System Switching Valve A Circuit Malfunction |
| P0413 | Secondary Air Injection System Switching Valve A Circuit Open |
| P0414 | Secondary Air Injection System Switching Valve A Circuit Shorted |
| P0415 | Secondary Air Injection System Switching Valve B Circuit Malfunction |
| P0416 | Secondary Air Injection System Switching Valve B Circuit Open |
| P0417 | Secondary Air Injection System Switching Valve B Circuit Shorted |
| P0418 | Secondary Air Injection System Relay "A" Circuit Malfunction |
| P0419 | Secondary Air Injection System Relay "B" Circuit Malfunction |
| P0420 | Catalyst System Efficiency Below Threshold (Bank 1) |
| P0421 | Warm Up Catalyst Efficiency Below Threshold (Bank 1) |
| P0422 | Main Catalyst Efficiency Below Threshold (Bank 1) |
| P0423 | Heated Catalyst Efficiency Below Threshold (Bank 1) |
| P0424 | Heated Catalyst Temperature Below Threshold (Bank 1) |
| P0426 | Catalyst Temperature Sensor Range/Performance (Bank 1) |
| P0427 | Catalyst Temperature Sensor Low Input (Bank 1) |
| P0428 | Catalyst Temperature Sensor High Input (Bank 1) |
| P0430 | Catalyst System Efficiency Below Threshold (Bank 2) |
| P0431 | Warm Up Catalyst Efficiency Below Threshold (Bank 2) |
| P0432 | Main Catalyst Efficiency Below Threshold (Bank 2) |
| P0433 | Heated Catalyst Efficiency Below Threshold (Bank 2) |
| P0434 | Heated Catalyst Temperature Below Threshold (Bank 2) |
| P0436 | Catalyst Temperature Sensor Range/Performance (Bank 2) |
| P0437 | Catalyst Temperature Sensor Low Input (Bank 2) |
| P0438 | Catalyst Temperature Sensor High Input (Bank 2) |
| P0440 | Evaporative Emission Control System Malfunction |
| P0441 | Evaporative Emission Control System Incorrect Purge Flow |
| P0442 | Evaporative Emission Control System Leak Detected (small leak) |
| P0443 | Evaporative Emission Control System Purge Control Valve Circuit Malfunction |
| P0444 | Evaporative Emission Control System Purge Control Valve Circuit Open |
| P0445 | Evaporative Emission Control System Purge Control Valve Circuit Shorted |
| P0446 | Evaporative Emission Control System Vent Control Circuit Malfunction |
| P0447 | Evaporative Emission Control System Vent Control Circuit Open |
| P0448 | Evaporative Emission Control System Vent Control Circuit Shorted |
| P0449 | Evaporative Emission Control System Vent Valve/Solenoid Circuit Malfunction |
| P0450 | Evaporative Emission Control System Pressure Sensor Malfunction |
| P0451 | Evaporative Emission Control System Pressure Sensor Range/Performance |
| P0452 | Evaporative Emission Control System Pressure Sensor Low Input |
| P0453 | Evaporative Emission Control System Pressure Sensor High Input |
| P0454 | Evaporative Emission Control System Pressure Sensor Intermittent |
| P0455 | Evaporative Emission Control System Leak Detected (gross leak) |
| P0460 | Fuel Level Sensor Circuit Malfunction |
| P0461 | Fuel Level Sensor Circuit Range/Performance |
| P0462 | Fuel Level Sensor Circuit Low Input |
| P0463 | Fuel Level Sensor Circuit High Input |
| P0464 | Fuel Level Sensor Circuit Intermittent |
| P0465 | Purge Flow Sensor Circuit Malfunction |
| P0466 | Purge Flow Sensor Circuit Range/Performance |
| P0467 | Purge Flow Sensor Circuit Low Input |
| P0468 | Purge Flow Sensor Circuit High Input |
| P0469 | Purge Flow Sensor Circuit Intermittent |
| P0470 | Exhaust Pressure Sensor Malfunction |
| P0471 | Exhaust Pressure Sensor Range/Performance |
| P0472 | Exhaust Pressure Sensor Low |
| P0473 | Exhaust Pressure Sensor High |
| P0474 | Exhaust Pressure Sensor Intermittent |
| P0475 | Exhaust Pressure Control Valve Malfunction |
| P0476 | Exhaust Pressure Control Valve Range/Performance |
| P0477 | Exhaust Pressure Control Valve Low |
| P0478 | Exhaust Pressure Control Valve High |
| P0479 | Exhaust Pressure Control Valve Intermittent |
| P0480 | Cooling Fan 1 Control Circuit Malfunction |
| P0481 | Cooling Fan 2 Control Circuit Malfunction |
| P0482 | Cooling Fan 3 Control Circuit Malfunction |
| P0483 | Cooling Fan Rationality Check Malfunction |
| P0484 | Cooling Fan Circuit Over Current |
| P0485 | Cooling Fan Power/Ground Circuit Malfunction |
| P0500 | Vehicle Speed Sensor Malfunction |
| P0501 | Vehicle Speed Sensor Range/Performance |
| P0502 | Vehicle Speed Sensor Low Input |
| P0503 | Vehicle Speed Sensor Intermittent/Erratic/High |
| P0505 | Idle Control System Malfunction |
| P0506 | Idle Control System RPM Lower Than Expected |
| P0507 | Idle Control System RPM Higher Than Expected |
| P0510 | Closed Throttle Position Switch Malfunction |
| P0520 | Engine Oil Pressure Sensor/Switch Circuit Malfunction |
| P0521 | Engine Oil Pressure Sensor/Switch Circuit Range/Performance |
| P0522 | Engine Oil Pressure Sensor/Switch Circuit Low Voltage |
| P0523 | Engine Oil Pressure Sensor/Switch Circuit High Voltage |
| P0530 | A/C Refrigerant Pressure Sensor Circuit Malfunction |
| P0531 | A/C Refrigerant Pressure Sensor Circuit Range/Performance |
| P0532 | A/C Refrigerant Pressure Sensor Circuit Low Input |
| P0533 | A/C Refrigerant Pressure Sensor Circuit High Input |
| P0534 | Air Conditioner Refrigerant Charge Loss |
| P0550 | Power Steering Pressure Sensor Circuit Malfunction |
| P0551 | Power Steering Pressure Sensor Circuit Range/Performance |
| P0552 | Power Steering Pressure Sensor Circuit Low Input |
| P0553 | Power Steering Pressure Sensor Circuit High Input |
| P0554 | Power Steering Pressure Sensor Circuit Intermittent |
| P0560 | System Voltage Malfunction |
| P0561 | System Voltage Unstable |
| P0562 | System Voltage Low |
| P0562 | A/C pressure sensor low voltage(Chrysler) |
| P0563 | System Voltage High |
| P0565 | Cruise Control On Signal Malfunction |
| P0566 | Cruise Control Off Signal Malfunction |
| P0567 | Cruise Control Resume Signal Malfunction |
| P0568 | Cruise Control Set Signal Malfunction |
| P0569 | Cruise Control Coast Signal Malfunction |
| P0570 | Cruise Control Accel Signal Malfunction |
| P0571 | Cruise Control/Brake Switch A Circuit Malfunction |
| P0572 | Cruise Control/Brake Switch A Circuit Low |
| P0573 | Cruise Control/Brake Switch A Circuit High |
| P0574 | Cruise Control Related Malfunction |
| P0575 | Cruise Control Related Malfunction |
| P0576 | Cruise Control Related Malfunction |
| P0576 | Cruise Control Related Malfunction |
| P0578 | Cruise Control Related Malfunction |
| P0579 | Cruise Control Related Malfunction |
| P0580 | Cruise Control Related Malfunction |
| P0600 | Serial Communication Link Malfunction |
| P0601 | Control Module Read Only Memory(ROM) |
| P0602 | Control Module Programming Error |
| P0603 | Internal Control Module Keep Alive Memory (KAM) Error |
| P0604 | Internal Control Module Random Access Memory (RAM) Error |
| P0605 | Internal Control Module Read Only Memory (ROM) Error |
| P0606 | PCM Processor Fault |
| P0608 | Control Module VSS Output "A" Malfunction |
| P0609 | Control Module VSS Output "B" Malfunction |
| P0620 | Generator Control Circuit Malfunction |
| P0621 | Generator Lamp "L" Control Circuit Malfunction |
| P0622 | Generator Field "F" Control Circuit Malfunction |
| P0650 | Malfunction Indicator Lamp (MIL) Control Circuit Malfunction |
| P0654 | Engine RPM Output Circuit Malfunction |
| P0655 | Engine Hot Lamp Output Control Circuit Malfucntion |
| P0656 | Fuel Level Output Circuit Malfunction |
| P0700 | Transmission Control System Malfunction |
| P0701 | Transmission Control System Range/Performance |
| P0702 | Transmission Control System Electrical |
| P0703 | Torque Converter/Brake Switch B Circuit Malfunction |
| P0704 | Clutch Switch Input Circuit Malfunction |
| P0705 | Transmission Range Sensor Circuit malfunction (PRNDL Input) |
| P0706 | Transmission Range Sensor Circuit Range/Performance |
| P0707 | Transmission Range Sensor Circuit Low Input |
| P0708 | Transmission Range Sensor Circuit High Input |
| P0709 | Transmission Range Sensor Circuit Intermittent |
| P0710 | Transmission Fluid Temperature Sensor Circuit Malfunction |
| P0711 | Transmission Fluid Temperature Sensor Circuit Range/Performance |
| P0712 | Transmission Fluid Temperature Sensor Circuit Low Input |
| P0713 | Transmission Fluid Temperature Sensor Circuit High Input |
| P0714 | Transmission Fluid Temperature Sensor Circuit Intermittent |
| P0715 | Input/Turbine Speed Sensor Circuit Malfunction |
| P0716 | Input/Turbine Speed Sensor Circuit Range/Performance |
| P0717 | Input/Turbine Speed Sensor Circuit No Signal |
| P0718 | Input/Turbine Speed Sensor Circuit Intermittent |
| P0719 | Torque Converter/Brake Switch B Circuit Low |
| P0720 | Output Speed Sensor Circuit Malfunction |
| P0721 | Output Speed Sensor Range/Performance |
| P0722 | Output Speed Sensor No Signal |
| P0723 | Output Speed Sensor Intermittent |
| P0724 | Torque Converter/Brake Switch B Circuit High |
| P0725 | Engine Speed input Circuit Malfunction |
| P0726 | Engine Speed Input Circuit Range/Performance |
| P0727 | Engine Speed Input Circuit No Signal |
| P0728 | Engine Speed Input Circuit Intermittent |
| P0730 | Incorrect Gear Ratio |
| P0731 | Gear 1 Incorrect ratio |
| P0732 | Gear 2 Incorrect ratio |
| P0733 | Gear 3 Incorrect ratio |
| P0734 | Gear 4 Incorrect ratio |
| P0735 | Gear 5 Incorrect ratio |
| P0736 | Reverse incorrect gear ratio |
| P0740 | Torque Converter Clutch Circuit Malfuction |
| P0741 | Torque Converter Clutch Circuit Performance or Stuck Off |
| P0742 | Torque Converter Clutch Circuit Stuck On |
| P0743 | Torque Converter Clutch Circuit Electrical |
| P0744 | Torque Converter Clutch Circuit Intermittent |
| P0745 | Pressure Control Solenoid Malfunction |
| P0746 | Pressure Control Solenoid Performance or Stuck Off |
| P0747 | Pressure Control Solenoid Stuck On |
| P0748 | Pressure Control Solenoid Electrical |
| P0749 | Pressure Control Solenoid Intermittent |
| P0750 | Shift Solenoid A Malfunction |
| P0751 | Shift Solenoid A Performance or Stuck Off |
| P0752 | Shift Solenoid A Stuck On |
| P0753 | Shift Solenoid A Electrical |
| P0754 | Shift Solenoid A Intermittent |
| P0755 | Shift Solenoid B Malfunction |
| P0756 | Shift Solenoid B Performance or Stuck Off |
| P0757 | Shift Solenoid B Stuck On |
| P0758 | Shift Solenoid B Electrical |
| P0759 | Shift Solenoid B Intermittent |
| P0760 | Shift Solenoid C Malfunction |
| P0761 | Shift Solenoid C Performance or Stuck Off |
| P0762 | Shift Solenoid C Stuck On |
| P0763 | Shift Solenoid C Electrical |
| P0764 | Shift Solenoid C Intermittent |
| P0765 | Shift Solenoid D Malfunction |
| P0766 | Shift Solenoid D Performance or Stuck Off |
| P0767 | Shift Solenoid D Stuck On |
| P0768 | Shift Solenoid D Electrical |
| P0769 | Shift Solenoid D Intermittent |
| P0770 | Shift Solenoid E Malfunction |
| P0771 | Shift Solenoid E Performance or Stuck Off |
| P0772 | Shift Solenoid E Stuck On |
| P0773 | Shift Solenoid E Electrical |
| P0774 | Shift Solenoid E Intermittent |
| P0775 | Pressure Control Solenoid B Malfunction |
| P0776 | Pressure Control Solenoid B Performance |
| P0777 | Pressure Control Solenoid B Stuck On |
| P0778 | Pressure Control Solenoid B Electrical |
| P0779 | Pressure Control Solenoid B Intermittent |
| P0780 | Shift Malfunction |
| P0781 | 1-2 Shift Malfunction |
| P0782 | 2-3 Shift Malfunction |
| P0783 | 3-4 Shift Malfunction |
| P0784 | 4-5 Shift Malfunction |
| P0785 | Shift/Timing Solenoid Malfunction |
| P0786 | Shift/Timing Solenoid Range/Performance |
| P0787 | Shift/Timing Solenoid Low |
| P0788 | Shift/Timing Solenoid High |
| P0789 | Shift/Timing Solenoid Intermittent |
| P0790 | Normal/Performance Switch Circuit Malfunction |
| P0801 | Reverse Inhibit Control Circuit Malfunction |
| P0803 | 1-4 Upshift (Skip Shift) Solenoid Control Circuit Malfunction |
| P0804 | 1-4 Upshift (Skip Shift) Lamp Control Circuit Malfunction |
| P1000 | Check of all OBDII Systems Not Complete |
| P1101 | KOER Not Able To Complete KOER Aborted |
| P1106 | Manifold Absolute Pressure (MAP) Sensor Circuit Intermittent High Voltage |
| P1107 | Manifold Absolute Pressure (MAP) Sensor Circuit Intermittent Low Voltage |
| P1100 | MAF Sensor Intermittent |
| P1101 | MAF Sensor Out Of Self Test Range |
| P1102 | MAF Sensor In Range But Lower Than Expected |
| P1103 | MAF Sensor In Range But Higher Than Expected |
| P1104 | MAF Ground Malfunction |
| P1105 | Dual Alternator Upper Fault |
| P1106 | Dual Alternator Lower Fault |
| P1107 | Dual Alternator Lower Circuit Malfunction |
| P1108 | Dual Alternator Battery Lamp Circuit Malfunction |
| P1109 | IAT - B Sensor Intermittent |
| P1110 | IAT Sensor (D/C) Open/Short |
| P1111 | Intake Air Temperature (IAT) Sensor Circuit Intermittent High Voltage |
| P1112 | Intake Air Temperature (IAT) Sensor Circuit Intermittent Low Voltage |
| P1113 | IAT Sensor Open/Short |
| P1114 | Engine Coolant Temperature (ECT) Sensor Circuit Intermittent Low Voltage/IAT - B Circuit Low Input |
| P1115 | Engine Coolant Temperature (ECT) Sensor Circuit Intermittent High Voltage/IAT - B Circuit High Input |
| P1116 | Engine Coolant sensor out of range/ECT Sensor Out Of Self Test Range |
| P1117 | Engine Coolant Sensor intermittent/ECT Sensor Intermittent |
| P1118 | Manifold Absolute Temperature Circuit Low Input |
| P1119 | Manifold Absolute Temperature Circuit High Input |
| P1120 | Throttle position sensor out of range |
| P1121 | Throttle Position (TP) Sensor Circuit Intermittent High Voltage |
| P1122 | Throttle Position (TP) Sensor Circuit Intermittent Low Voltage |
| P1123 | Throttle Position Sensor In Range But Higher Than Expected |
| P1124 | Throttle Position Sensor Out Of Self Test Range |
| P1125 | Throttle position sensor intermittent |
| P1126 | Throttle Position (Narrow Range) Sensor Circuit Malfunction |
| P1127 | Exhaust Not Warm, Downstream O2 Sensor |
| P1128 | Upstream Heated O2 Sensors Swapped |
| P1129 | Downstream Heated O2 Sensors Swapped |
| P1130 | Lack Of HO2S Switch - Adaptive Fuel At Limit |
| P1131 | Lack Of HO2S Switch - Sensor Indicates Lean |
| P1132 | Lack Of HO2S Switch - Sensor Indicates Rich |
| P1133 | HO2S Insufficient Switching Sensor 1 |
| P1134 | HO2S Transition Time Ratio Sensor 1 |
| P1135 | Pedal Position Sensor A Circuit Intermittent |
| P1136 | Fan Control Circuit Malfunction |
| P1137 | Lack Of HO2S Switch - Sensor Indicates Lean |
| P1138 | Lack Of HO2S12 Switch - Sensor Indicates Rich |
| P1139 | Water In Fuel Indicator Circuit Malfunction |
| P1140 | Water In Fuel Condition |
| P1141 | Fuel Restriction Indicator Circuit Malfunction |
| P1142 | Fuel Restriction Condition |
| P1143 | Air Assist Control Valve Range/Performance |
| P1144 | Air Assist Control Valve Circuit Malfunction |
| P1150 | Lack Of HO2S21 Switch - Adaptive Fuel At Limit |
| P1151 | Lack Of HO2S21 Switch - Sensor Indicates Lean |
| P1152 | Lack Of HO2S21 Switch - Sensor Indicates Rich |
| P1153 | Bank 2 Fuel Control Shifted Lean |
| P1154 | Bank 2 Fuel Control Shifted Rich |
| P1155 | Alternative Fuel Controller |
| P1156 | Fuel Select Switch Malfunction |
| P1157 | Lack Of HO2S22 Switch - Sensor Indicates Lean |
| P1158 | Lack Of HO2S22 Switch - Sensor Indicates Rich |
| P1159 | Fuel Stepper Motor Malfunction |
| P1167 | Invalid Test,throttle not depressed |
| P1168 | Fuel Rail Sensor In-Range Low Failure |
| P1169 | Fuel Rail Sensor In-Range High Failure |
| P1170 | ESO - Engine Shut Off Solenoid Fault |
| P1171 | Rotor Sensor Fault |
| P1172 | Rotor Control Fault |
| P1173 | Rotor Calibration Fault |
| P1174 | Cam Sensor Fault |
| P1175 | Cam Control Fault |
| P1176 | Cam Calibration Fault |
| P1177 | Synchronization Fault |
| P1178 | ( open ) |
| P1180 | Fuel Delivery System Malfunction - Low |
| P1181 | Fuel Delivery System Malfunction - High |
| P1182 | Fuel Shut Off Solenoid Malfunction |
| P1183 | Engine Oil Temperature Circuit Malfunction |
| P1184 | Engine Oil Temperature Out Of Self Test Range |
| P1185 | FTS High - Fuel Pump Temperature Sensor High |
| P1186 | FTS Low - Fuel Pump Temperature Sensor Low |
| P1187 | Variant Selection |
| P1188 | Calibration Memory Fault |
| P1189 | Pump Speed Signal Fault |
| P1190 | Calibration Resistor Out Of Range |
| P1191 | Key Line Voltage |
| P1192 | Voltage External |
| P1193 | EGR Drive Overcurrent |
| P1194 | ECU A/D Converter |
| P1195 | SCP HBCC Failed To Initialize |
| P1196 | Key Off Voltage High |
| P1197 | Key Off Voltage Low |
| P1198 | Pump Rotor Control Underfueling |
| P1199 | Fuel Level Input Circuit Low |
| P1200 | Injector Control Circuit |
| P1201 | Injector Circuit Open / Shorted - Cylinder #1 |
| P1202 | Injector Circuit Open / Shorted - Cylinder #2 |
| P1203 | Injector Circuit Open / Shorted - Cylinder #3 |
| P1204 | Injector Circuit Open / Shorted - Cylinder #4 |
| P1205 | Injector Circuit Open / Shorted - Cylinder #5 |
| P1206 | Injector Circuit Open / Shorted - Cylinder #6 |
| P1209 | Injector Control Pressure System Fault |
| P1210 | Injector Control Pressure Above Expected Level |
| P1211 | Injector Control Pressure Sensor Above / Below Desired |
| P1212 | Injector Control Pressure Not Detected During Crank |
| P1213 | Start Injector Circuit Malfunction |
| P1214 | Pedal Position Sensor B Circuit Intermittent |
| P1215 | Pedal Position Sensor C Circuit Low Input |
| P1216 | Pedal Position Sensor C Circuit High Input |
| P1217 | Pedal Position Sensor C Circuit Intermittent |
| P1218 | CID High |
| P1219 | CID Low |
| P1220 | Series Throttle Control System Malfunction |
| P1221 | Traction Control System Malfunction |
| P1222 | Traction Control Output Circuit Malfunction |
| P1223 | Pedal Demand Sensor B Circuit High Input |
| P1224 | Throttle Position Sensor B Out Of Self Test Range |
| P1225 | Needle Lift Sensor Malfunction |
| P1226 | Control Sleeve Sensor Malfunction |
| P1227 | Wastegate Failed Closed (Over Pressure) |
| P1228 | Wastegate Failed Open (Under Pressure) |
| P1229 | Intercooler Pump Driver Fault |
| P1230 | Fuel Pump Low Speed Malfunction |
| P1231 | Fuel Pump Secondary Circuit Low, High Speed |
| P1232 | Fuel Pump Speed Primary Circuit Malfunction |
| P1233 | Fuel Pump Driver Module Off Line |
| P1234 | Fuel Pump Driver Module Off Line |
| P1235 | Fuel Pump Control Out Of Range |
| P1236 | Fuel Pump Control Out Of Range |
| P1237 | Fuel Pump Secondary Circuit Malfunction |
| P1238 | Fuel Pump Secondary Circuit Malfunction |
| P1239 | Speed Fuel Pump Positive Feed Fault |
| P1240 | Sensor Power Supply Malfunction |
| P1241 | Sensor Power Supply Low Input |
| P1242 | Sensor Power Supply High Input |
| P1243 | Second Fuel Pump Faulty or Ground Fault |
| P1244 | Alternator Load Input Failed High |
| P1245 | Alternator Load Input Failed Low |
| P1246 | Alternator Load Input Failed |
| P1247 | Turbo Boost Pressure Low |
| P1248 | Turbo Boost Pressure Not Detected |
| P1249 | Wastegate Control Valve Performance |
| P1250 | PRC Solenoid Circuit Malfunction |
| P1251 | Air Mixture Solenoid Circuit Malfunction |
| P1252 | Pedal Correlation PDS1 and LPDS High |
| P1253 | Pedal Correlation PDS1 and LPDS Low |
| P1254 | Pedal Correlation PDS2 and LPDS High |
| P1255 | Pedal Correlation PDS2 and LPDS Low |
| P1256 | Pedal Correlation PDS1 and HPDS |
| P1257 | Pedal Correlation PDS2 and HPDS |
| P1258 | Pedal Correlation PDS1 and PDS2 |
| P1259 | Immobilizer to PCM Signal Error |
| P1260 | THEFT Detected, Vehicle Immobilzed |
| P1261 | Cylinder #1 High To Low Side Short |
| P1262 | Cylinder #2 High To Low Side Short |
| P1263 | Cylinder #3 High To Low Side Short |
| P1264 | Cylinder #4 High To Low Side Short |
| P1265 | Cylinder #5 High To Low Side Short |
| P1266 | Cylinder #6 High To Low Side Short |
| P1267 | Cylinder #7 High To Low Side Short |
| P1268 | Cylinder #8 High To Low Side Short |
| P1269 | Immobilizer Code Not Programmed |
| P1270 | Engine RPM Or Speed Limiter Reached |
| P1271 | Cylinder #1 High To Low Side Open |
| P1272 | Cylinder #2 High To Low Side Open |
| P1273 | Cylinder #3 High To Low Side Open |
| P1274 | Cylinder #4 High To Low Side Open |
| P1275 | Cylinder #5 High To Low Side Open |
| P1276 | Cylinder #6 High To Low Side Open |
| P1277 | Cylinder #7 High To Low Side Open |
| P1278 | Cylinder #8 High To Low Side Open |
| P1280 | Injection Control Pressure Out Of Range Low |
| P1281 | Injection Control Pressure Out Of Range High |
| P1282 | Excessive Injection Control Pressure |
| P1283 | IPR Circuit Failure |
| P1284 | Aborted KOER - ICP Failure |
| P1285 | Cylinder head over temp sensed |
| P1286 | Fuel Pulse In Range But Lower Than Expected |
| P1287 | Fuel Pulse In Range But Higher Than Expected |
| P1288 | Cylinder Head Temp Sensor Out Of Self Test Range |
| P1289 | Cylinder Head Temp Sensor High Input |
| P1290 | Cylinder Head Temp Sensor Low Input |
| P1291 | Injector High Side Short To GND Or VBATT - Bank 1 |
| P1292 | Injector High Side Short To GND Or VBATT - Bank 2 |
| P1293 | Injector High Side Open - Bank 1 |
| P1294 | Injector High Side Open - Bank 2/Target idle not reached |
| P1295 | Multi-faults - Bank 1 - With Low Side Shorts |
| P1296 | Multi-faults - Bank 2 - With Low Side Shorts |
| P1297 | Injector High Sides Shorted Together |
| P1298 | IDM Failure |
| P1299 | Cylinder Head Overtemperature Protection Active |
| P1300 | Boost Calibration Fault |
| P1301 | Boost Calibration High |
| P1302 | Boost Calibration Low |
| P1303 | EGR Calibration Fault |
| P1304 | EGR Calibration High |
| P1305 | EGR Calibration Low |
| P1306 | Kickdown Relay Pull - In Circuit Fault |
| P1307 | Kickdown Relay Hold Circuit Fault |
| P1308 | A/C Clutch Circuit Fault |
| P1309 | Misfire Monitor AICE Chip Fault |
| P1313 | Misfire Rate Catalyst Damage Fault - Bank 1 |
| P1314 | Misfire Rate Catalyst Damage Fault - Bank 2 |
| P1315 | Persistent Misfire |
| P1316 | Injector Circuit / IDM Codes Detected |
| P1317 | Injector Circuit / IDM Codes Not Updated |
| P1336 | Crank / Cam Sensor Range / Performance |
| P1340 | Camshaft Position Sensor B Circuit Malfunction |
| P1341 | Camshaft Position Sensor B Range / Performance |
| P1345 | SGC (Cam Position) Sensor Circuit Malfunction |
| P1346 | Fuel Level Sensor B Circuit Malfunction |
| P1347 | Fuel Level Sensor B Range / Performance |
| P1348 | Fuel Level Sensor B Circuit Low |
| P1349 | Fuel Level Sensor B Circuit High |
| P1350 | Fuel Level Sensor B Intermittent/Bypass Line Monitor |
| P1351 | IDM Input Circuit Malfunction |
| P1352 | Ignition Coil A Primary Circuit Malfunction |
| P1353 | Ignition Coil B Primary Circuit Malfunction |
| P1354 | Ignition Coil C Primary Circuit Malfunction |
| P1355 | Ignition Coil D Primary Circuit Malfunction |
| P1360 | Ignition Coil A Secondary Circuit Malfunction |
| P1361 | Ignition Control (IC) Circuit Low Voltage |
| P1362 | Ignition Coil C Secondary Circuit Malfunction |
| P1363 | Ignition Coil D Secondary Circuit Malfunction |
| P1364 | Ignition Coil Primary Circuit Failure |
| P1365 | Ignition Coil Secondary Circuit Failure |
| P1366 | Ignition Spare |
| P1367 | Ignition Spare |
| P1368 | Ignition Spare |
| P1369 | Engine Temperature Light Monitor Failure |
| P1370 | Insufficient RMP Increase During Spark Test |
| P1371 | Ignition Coil - Cylinder 1 - Early Activation Fault |
| P1372 | Ignition Coil - Cylinder 2 - Early Activation Fault |
| P1373 | Ignition Coil - Cylinder 3 - Early Activation Fault |
| P1374 | Crankshaft Position (CKP)/Ignition Coil - Cylinder 4 - Early Activation Fault |
| P1375 | Ignition Coil - Cylinder 5 - Early Activation Fault |
| P1376 | Ignition Coil - Cylinder 6 - Early Activation Fault |
| P1380 | Misfire Detected - Rough Road Data Not Available |
| P1381 | Variable Cam Timing Overadvanced (Bank #1) |
| P1382 | Variable Cam Timing Solenoid #1 Circuit Malfunction |
| P1383 | Variable Cam Timing Overretarded (Bank #1) |
| P1384 | VVT Solenoid A Malfunction |
| P1385 | Variable Cam Timing Solenoid B Malfunction |
| P1386 | Variable Cam Timing Overadvanced (Bank #2) |
| P1387 | Variable Cam Timing Solenoid #2 Circuit Malfunction |
| P1388 | Variable Cam Timing Overretarded (Bank #2) |
| P1389 | Glow Plug Circuit High Side Low Input |
| P1390 | Octane Adjust Pin Out Of Self Test Range |
| P1391 | Glow Plug Circuit Low Input (Bank #1) |
| P1392 | Glow Plug Circuit High Input (Bank #1) |
| P1393 | Glow Plug Circuit Low Input (Bank #2) |
| P1394 | Glow Plug Circuit High Input (Bank #2) |
| P1395 | Glow Plug Monitor Fault (Bank #1) |
| P1396 | Glow Plug Monitor Fault (Bank #2) |
| P1397 | System Voltage Out Of Self Test Range |
| P1398 | VVT Solenoid B Circuit High Input |
| P1399 | Glow Plug Circuit High Side, High Input |
| P1400 | DPFE Circuit Low Input |
| P1401 | DPFE Circuit High Input |
| P1402 | EGR Metering Orifice Restricted |
| P1403 | DPFE Sensor Hoses Reversed |
| P1404 | IAT - B Circuit Malfunction |
| P1405 | DPFE Sensor Upstream Hose Off Or Plugged |
| P1406 | Exhaust Gas Recirculation (EGR) Position Sensor Performance |
| P1407 | EGR No Flow Detected |
| P1408 | EGR Flow Out Of Self Test Range |
| P1409 | EVR Control Circuit Malfunction |
| P1411 | SAI System Incorrect Downstream Flow Detected |
| P1413 | SAI System Monitor Circuit Low Input |
| P1414 | SAI System Monitor Circuit High Input |
| P1415 | Air Pump Circuit Malfunction |
| P1416 | Port Air Circuit Malfunction |
| P1417 | Port Air Relief Circuit Malfunction |
| P1418 | Split Air #1 Circuit Malfunction |
| P1419 | Split Air #2 Circuit Malfunction |
| P1420 | Catalyst Temperature Sensor Failure |
| P1421 | Catalyst Damage |
| P1422 | EGI Temperature Sensor Failure |
| P1423 | EGI Functionality Test Failed |
| P1424 | EGI Glow Plug Primary Failure |
| P1425 | EGI Glow Plug Secondary Failure |
| P1426 | EGI Mini - MAF Failed Out Of Range |
| P1427 | EGI Mini - MAF Failed Short Circuit |
| P1428 | EGI Mini - MAF Failed Open Circuit |
| P1429 | Electric Air Pump Primary Failure |
| P1430 | Electric Air Pump Secondary Failure |
| P1433 | A/C Refrigerant Temperature Circuit Low |
| P1434 | A/C Refrigerant Temperature Circuit High |
| P1435 | A/C Refrigerant Temperature Circuit Range/Performance |
| P1436 | A/C Evaporator Air Temperature Circuit Low |
| P1437 | A/C Evaporator Air Temperature Circuit High |
| P1438 | A/C Evaporator Air Temperature Circuit Range/Performance |
| P1439 | Floor Temperature Switch Circuit Malfunction |
| P1440 | Purge Valve Stuck Open |
| P1441 | Evaporative Emission (EVAP) System Flow During Non-Purge Chevrolet Only |
| P1441 | Evaporative Emission (EVAP) System Flow During Non-Purge Oldsmobile Only |
| P1442 | Evaporative Emission Control System Leak Detected |
| P1443 | Evaporative Emission Control System Control Valve |
| P1444 | Purge Flow Sensor Circuit Low Input |
| P1445 | Purge Flow Sensor Circuit High Input |
| P1446 | Evaporative Vac Solenoid Circuit Malfunction |
| P1447 | ELC System Closure Valve Flow Fault |
| P1448 | ELC System 2 Fault |
| P1449 | Evaporative Check Solenoid Circuit Malfunction |
| P1450 | Unable To Bleed Up Fuel Tank Vacuum |
| P1451 | Evap Emission Control Sys Vent Control Valve Circuit |
| P1452 | Unable To Bleed - Up Vacuum in Tank |
| P1453 | Fuel Tank Pressure Relief Valve Malfunction |
| P1454 | Evaporative System Vacuum Test Malfunction |
| P1455 | Evap Emission Control Sys Leak Detected (Gross Leak/No Flow) |
| P1456 | Fuel Tank Temperature Sensor Circuit Malfunction |
| P1457 | Unable To Pull Vacuum In Tank |
| P1460 | Wide open throttle A/C cutoff relay circuit |
| P1461 | A/C pressure sensor circuit voltage low |
| P1462 | A/C pressure sensor circuit voltage high |
| P1463 | A/C Pressure Sensor Insufficient Pressure Change |
| P1464 | A/C Demand Out of Self Test Range |
| P1465 | A/C Relay Circuit Malfunction |
| P1466 | A/C Refrigerant Temperature Sensor/Circuit Malfunction |
| P1467 | A/C Compressor Temperature Sensor Malfunction |
| P1468 | SSPOD Open Circuit or Closed Circuit Fault |
| P1469 | Low A/C Cycling Period |
| P1470 | A/C Cycling Period Too Short |
| P1471 | Electrodrive Fan 1 Operational Failure (Driver Side) |
| P1472 | Electrodrive Fan 2 Operational Failure (Passenger Side) |
| P1473 | Fan Secondary High With Fan(s) Off |
| P1474 | Low Fan Control Primary Circuit Malfunction |
| P1475 | Fan Relay (Low) Circuit Malfunction |
| P1476 | Fan Relay (High) Circuit Malfunction |
| P1477 | Additional Fan Relay Circuit Malfunction |
| P1478 | Cooling Fan Driver Fault |
| P1479 | High Fan Control Primary Circuit Malfunction |
| P1480 | Fan Secondary Low with Low Fan On |
| P1481 | Fan Secondary Low With High Fan On |
| P1482 | SCP |
| P1483 | Power To Fan Circuit Overcurrent |
| P1484 | Open Power To Ground VCRM |
| P1485 | EGRV Circuit Malfunction |
| P1486 | EGRA Circuit Malfunction |
| P1487 | EGRCHK Solenoid Circuit Malfunction |
| P1490 | Secondary Air Relief Solenoid Circuit Malfunction |
| P1491 | Secondary Switch Solenoid Circuit Malfunction |
| P1492 | APLSOL Solenoid Circuit Malfunction |
| P1493 | RCNT Solenoid Circuit Malfunction |
| P1494 | SPCUT Solenoid Circuit Malfunction |
| P1495 | TCSPL Solenoid Circuit Malfunction |
| P1500 | Vehicle Speed Sensor Intermittent |
| P1501 | Vehicle Speed Sensor Out Of Self Test Range |
| P1502 | Vehicle Speed Sensor Intermittent Malfunction |
| P1503 | Auxillary Speed Sensor Fault |
| P1504 | Idle Air Control Circuit Malfunction |
| P1505 | Idle Air Control System At Adaptive Clip |
| P1506 | Idle Air Control Overspeed Error |
| P1507 | Idle Air Control Underspeed Error |
| P1508 | Idle Control System Circuit Open |
| P1509 | Idle Control System Circuit Shorted |
| P1510 | Idle Signal Circuit Malfunction |
| P1511 | Idle Switch (Electric Control Throttle) Circuit Malfunction |
| P1512 | Intake Manifold Runner Control (Bank 1) Stuck Closed |
| P1513 | Intake Manifold Runner Control (Bank 2) Stuck Closed |
| P1514 | High Load Neutral/Drive Fault |
| P1515 | Electric Current Circuit Malfunction |
| P1516 | IMRC Input Error (Bank 1) |
| P1517 | IMRC Input Error (Bank 2) |
| P1518 | Intake Manifold Runner Control (Stuck Open) |
| P1519 | Intake Manifold Runner Control (Stuck Closed) |
| P1520 | Intake Manifold Runner Control Circuit Malfunction |
| P1521 | Variable Intake Solenoid #1 Circuit Malfunction |
| P1522 | Variable Intake Solenoid #2 Circuit Malfunction |
| P1523 | IVC Solenoid Circuit Malfunction |
| P1524 | Variable Intake Solenoid System |
| P1525 | Air Bypass Valve System |
| P1526 | Air Bypass System |
| P1527 | Accelerate Warmup Solenoid Circuit Malfunction |
| P1528 | Subsidiary Throttle Valve Solenoid Circuit Malfunction |
| P1529 | SCAIR Solenoid Circuit Malfunction |
| P1530 | A/C Clutch Circuit Malfunction |
| P1531 | Invalid Test - Accelerator Pedal Movement |
| P1532 | IMCC Circuit Malfunction, Bank B |
| P1533 | AAI Circuit Malfunction |
| P1534 | Inertia Switch Activated |
| P1535 | Blower Fan Speed Circuit Range/Performance |
| P1536 | Parking Brake Switch Circuit Failure |
| P1537 | Intake Manifold Runner Control (Bank 1) Stuck Open |
| P1538 | Intake Manifold Runner Control (Bank 2) Stuck Open |
| P1539 | Power To A/C Clutch Circuit Overcurrent |
| P1540 | Air Bypass Valve Circuit Malfunction |
| P1549 | IMCC Circuit Malfunction, Bank B |
| P1550 | PSPS Out Of Self Test Range |
| P1565 | Speed Control Command Switch Out of Range High |
| P1566 | Speed Control Command Switch Out of Range Low |
| P1567 | Speed Control Output Circuit Continuity |
| P1568 | Speed Control Unable to Hold Speed |
| P1571 | Brake Switch Malfunction |
| P1572 | Brake Pedal Switch Circuit Malfunction |
| P1573 | Throttle Position Not Available |
| P1574 | Throttle Position Sensor Disagreement btwn Sensors |
| P1575 | Pedal Position Out of Self Test Range |
| P1576 | Pedal Position Not Available |
| P1577 | Pedal Position Sensor Disagreement btwn Sensors |
| P1578 | ETC Power Less Than Demand |
| P1579 | ETC In Power Limiting Mode |
| P1580 | Electronic Throttle Monitor PCM Override |
| P1581 | Electronic Throttle Monitor Malfunction |
| P1582 | Electronic Throttle Monitor Data Available |
| P1583 | Electronic Throttle Monitor Cruise Disable |
| P1584 | TCU Detected IPE Circuit Malfunction |
| P1585 | Throttle Control Unit Malfunction |
| P1586 | Throttle Control Unit Throttle Position Malfunction |
| P1587 | Throttle Control Unit Modulated Command Malfunction |
| P1588 | Throttle Control Unit Detected Loss of Return Spring |
| P1589 | TCU Unable To Control Desired Throttle Angle |
| P1600 | Loss of KAM Power; Open Circuit |
| P1601 | ECM/TCM Serial Communication Error |
| P1602 | Immobilizer/ECM Communication Error |
| P1603 | EEPROM Malfunction |
| P1604 | Code Word Unregestered |
| P1605 | Keep Alive Memory Test Failure |
| P1606 | ECM Control Relay O/P Circuit Malfunction |
| P1607 | MIL O/P Circuit Malfunction |
| P1608 | Internal ECM Malfunction |
| P1609 | Diagnostic Lamp Driver Fault |
| P1610 | SBDS Interactive Codes |
| P1611 | SBDS Interactive Codes |
| P1612 | SBDS Interactive Codes |
| P1613 | SBDS Interactive Codes |
| P1614 | SBDS Interactive Codes |
| P1615 | SBDS Interactive Codes |
| P1616 | SBDS Interactive Codes |
| P1617 | SBDS Interactive Codes |
| P1618 | SBDS Interactive Codes |
| P1619 | SBDS Interactive Codes |
| P1620 | SBDS Interactive Codes |
| P1621 | Immobilizer Code Words Do Not Match |
| P1622 | Immobilizer ID Does Not Match |
| P1623 | Immobilizer Code Word/ID Number Write Failure |
| P1624 | Anti Theft System |
| P1625 | B+ Supply To VCRM Fan Circuit Malfunction |
| P1626 | B+ Supply To VCRM A/C Circuit Malfunction |
| P1627 | Module Supply Voltage Out Of Range |
| P1628 | Module Ignition Supply Input Malfunction |
| P1629 | Internal Voltage Regulator Malfunction |
| P1630 | Internal Vref Malfunction |
| P1631 | Main Relay Malfunction (Power Hold) |
| P1632 | Smart Alternator Faults Sensor/Circuit Malfunction |
| P1633 | KAM Voltage Too Low |
| P1634 | Data Output Link Circuit Failure |
| P1635 | Tire / Axle Ratio Out of Acceptable Range |
| P1636 | Inductive Signature Chip Communication Error |
| P1637 | Can Link ECM/ABSCM Circuit / Network Malfunction |
| P1638 | Can Link ECM/INSTM Circuit / Network Malfunction |
| P1639 | Vehicle ID Block Corrupted or Not Programmed |
| P1640 | Powertrain DTCs Available in Another Module |
| P1641 | Fuel Pump Primary Circuit Failure |
| P1642 | Fuel Pump Monitor Circuit High Input |
| P1643 | Fuel Pump Monitor Circuit Low Input |
| P1644 | Fuel Pump Speed Control Circuit Malfunction |
| P1645 | Fuel Pump Resistor Switch Circuit Malfunction |
| P1650 | PSP Switch Out of Self Test Range |
| P1651 | PSP Switch Input Malfunction |
| P1652 | IAC Monitor Disabled by PSP Switch Failed On |
| P1653 | Power Steering Output Circuit Malfunction |
| P1654 | Recirculation Override Circuit Malfunction |
| P1655 | Starter Disable Circuit Malfunction |
| P1660 | Output Circuit Check Signal High |
| P1661 | Output Circuit Check Signal Low |
| P1662 | IDM_EN Circuit Failure |
| P1663 | Fuel Demand Command Signal Circuit Malfunction |
| P1667 | CI Circuit Malfunction |
| P1668 | PCM - IDM Communications Error |
| P1670 | Electronic Feedback Signal Not Detected |
| P1680 | Metering Oil Pump Malfunction |
| P1681 | Metering Oil Pump Malfunction |
| P1682 | Metering Oil Pump Malfunction |
| P1683 | Metering Oil Pump Temperature Sensor Circuit Malfunction |
| P1684 | Metering Oil Pump Position Sensor Circuit Malfunction |
| P1685 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1686 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1687 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1688 | Metering Oil Pump Stepping Motor Circuit Malfunction |
| P1689 | Oil Pressure Control Solenoid Circuit Malfunction |
| P1690 | Wastegate Solenoid Circuit Malfunction |
| P1691 | Turbo Pressure Control Solenoid Circuit Malfunction |
| P1692 | Turbo Control Solenoid Circuit Malfunction |
| P1693 | Turbo Charge Control Circuit Malfunction |
| P1694 | Turbo Charge Relief Circuit Malfunction |
| P1700 | Transmission Indeterminate Failure (Failed to Neutral) |
| P1701 | Reverse Engagement Error |
| P1702 | TRS Circuit Intermittent Malfunction |
| P1703 | Brake Switch Out Of Self Test Range |
| P1704 | Digital TRS Failed to Transition States in KOEO / KOER |
| P1705 | Not in P or N During KOEO / KOER |
| P1706 | High Vehicle Speed Observed in Park |
| P1707 | Transfer Case Neutral Indicator Hard Fault Present |
| P1708 | Clutch Switch Circuit Malfunction |
| P1709 | PNP Switch Out Of Self Test Range |
| P1711 | TFT Sensor Out Of Self Test Range |
| P1712 | Trans Torque Reduction Request Signal Malfunction |
| P1713 | TFT Sensor In Range Failure Low Value |
| P1714 | SSA Inductive Signature Malfunction |
| P1715 | SSB Inductive Signature Malfunction |
| P1716 | SSC Inductive Signature Malfunction |
| P1717 | SSD Inductive Signature Malfunction |
| P1718 | TFT Sensor In Range Failure High |
| P1720 | Vehicle Speed (Meter) Circuit Malfunction |
| P1721 | Gear 1 Incorrect Ratio |
| P1722 | Gear 2 Incorrect Ratio |
| P1723 | Gear 3 incorrect Ratio |
| P1724 | Gear 4 Incorrect Ratio |
| P1725 | Insufficient Engine Speed Increase During Self Test |
| P1726 | Insufficient Engine Speed Decrease During Self Test |
| P1727 | Coast Clutch Solenoid Inductive Signature Malfunction |
| P1728 | Transmission Slip Error |
| P1729 | 4x4 Low Switch Error |
| P1730 | Gear Control Malfunction 2,3,5 |
| P1731 | 1-2 Shift Malfunction |
| P1732 | 2-3 Shift Malfunction |
| P1733 | 3-4 Shift Malfunction |
| P1734 | Gear Control Malfunction |
| P1735 | First Gear Switch Circuit Malfunction |
| P1736 | Second Gear Switch Circuit Malfunction |
| P1737 | Lockup Solenoid System |
| P1738 | Shift Time Error |
| P1739 | Slip Solenoid System |
| P1740 | Torque Converter Clutch Inductive Signature Malfunction |
| P1741 | Torque Converter Clutch Control Error |
| P1742 | Torque Converter Clutch Solenoid Failed On |
| P1743 | Torque Converter Clutch Solenoid Failied On |
| P1744 | Torque Converter Clutch System Performance |
| P1745 | Line Pressure Solenoid System |
| P1746 | Pressure Control Solenoid "A" Open Circuit |
| P1747 | Pressure Control Solenoid "A" Short Circuit |
| P1748 | EPC Malfunction |
| P1749 | Pressure Control Solenoid Failed Low |
| P1751 | Shift Solenoid A Performance |
| P1754 | Coast Clutch Solenoid Circuit Malfunction |
| P1755 | Intermediate Speed Sensor (ISS) Malfunction |
| P1756 | Shift Solenoid B Performance |
| P1760 | Pressure Control Solenoid "A" Short Circuit |
| P1761 | Shift Solenoid C Performance |
| P1762 | Overdrive Band Failed Off |
| P1765 | Timing Solenoid Circuit Malfunction |
| P1767 | Torque Converter Clutch Circuit Malfunction |
| P1768 | Performance / Normal / Winter Mode Input Malfunction |
| P1769 | AG4 Transmission Torque Modulation Fault |
| P1770 | Clutch Solenoid Circuit Malfunction |
| P1775 | Transmission System MIL Fault |
| P1776 | Ignition Retard Request Duration Fault |
| P1777 | Ignition Retard Request Circuit Fault |
| P1778 | Transmission Reverse I/P Circuit Malfunction |
| P1779 | TCIL Circuit Malfunction |
| P1780 | Trans Control Switch (O/D Cancel) Out of Self Test Range |
| P1781 | 4X4 Switch Out of Self Test Range |
| P1782 | P/ES Circuit Out Of Self Test Range |
| P1783 | Transmission Overtemperature Condition |
| P1784 | Transmission Mechanical Failure - First And Reverse |
| P1785 | Transmission Mechanical Failure - First And Second |
| P1786 | 3-2 Downshift Error |
| P1787 | 2-1 Downshift Error |
| P1788 | Pressure Control Solenoid "B" Open Circuit |
| P1789 | Pressure Control Solenoid "B" Short Circuit |
| P1790 | TP (Mechanical) Circuit Malfunction |
| P1791 | TP (Electric) Circuit Malfunction |
| P1792 | Barometer Pressure Circuit Malfunction |
| P1793 | Intake Air Volume Circuit Malfunction |
| P1794 | Battery Voltage Circuit Malfunction |
| P1795 | Idle Switch Circuit Malfunction |
| P1796 | Kick Down Switch Circuit Malfunction |
| P1797 | Neutral Switch Circuit Malfunction |
| P1798 | Coolant Temperature Circuit Malfunction |
| P1799 | Hold Switch Circuit Malfunction |
| P1800 | Transmission Clutch Interlock Safety Switch Circuit Failure |
| P1801 | Transmission Clutch Interlock Safety Switch Open Circuit |
| P1802 | Transmission Clutch Interlock Safety Switch Short Circuit To Battery |
| P1803 | Transmission Clutch Interlock Safety Switch Short Circuit To Ground |
| P1804 | Transmission 4-Wheel Drive High Indicator Circuit Failure |
| P1805 | Transmission 4-Wheel Drive High Indicator Open Circuit |
| P1806 | Transmission 4-Wheel Drive High Indicator Short Circuit To Battery |
| P1807 | Transmission 4-Wheel Drive High Indicator Short Circuit To Ground |
| P1808 | Transmission 4-Wheel Drive Low Indicator Circuit Failure |
| P1809 | Transmission 4-Wheel Drive Low Indicator Open Circuit |
| P1810 | Transmission 4-Wheel Drive Low Indicator Short Circuit To Battery |
| P1811 | Transmission 4-Wheel Drive Low Indicator Short Circuit To Ground |
| P1812 | Transmission 4-Wheel Drive Mode Select Circuit Failure |
| P1813 | Transmission 4-Wheel Drive Mode Select Open Circuit |
| P1814 | Transmission 4-Wheel Drive Mode Select Short Circuit To Battery |
| P1815 | Transmission 4-Wheel Drive Mode Select Short Circuit To Ground |
| P1816 | Transmission Neutral Safety Switch Circuit Failure |
| P1817 | Transmission Neutral Safety Switch Open Circuit |
| P1818 | Transmission Neutral Safety Switch Short Circuit To Battery |
| P1819 | Transmission Neutral Safety Switch Short Circuit To Ground |
| P1820 | Transmission Transfer Case Clockwise Shift Relay Coil Circuit Failure |
| P1821 | Transmission Transfer Case Clockwise Shift Relay Coil Open Circuit |
| P1822 | Transmission Transfer Case Clockwise Shift Relay Coil Short Circuit To Battery |
| P1823 | Transmission Transfer Case Clockwise Shift Relay Coil Short Circuit To Ground |
| P1824 | Transmission 4-Wheel Drive Clutch Relay Circuit Failure |
| P1825 | Transmission 4-Wheel Drive Clutch Relay Open Circuit |
| P1826 | Transmission 4-Wheel Drive Low Clutch Relay Circuit To Battery |
| P1827 | Transmission 4-Wheel Drive Low Clutch Relay Circuit To Ground |
| P1828 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Circuit Failure |
| P1829 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Open Circuit |
| P1830 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Short Circuit To Battery |
| P1831 | Transmission Transfer Case Counter Clockwise Shift Relay Coil Short Circuit To Ground |
| P1832 | Transmission Transfer Case Differential Lock-Up Solenoid Circuit Failure |
| P1833 | Transmission Transfer Case Differential Lock-Up Solenoid Open Circuit |
| P1834 | Transmission Transfer Case Differential Lock-Up Solenoid Short Circuit To Battery |
| P1835 | Transmission Transfer Case Differential Lock-Up Solenoid Short Circuit To Ground |
| P1836 | Transmission Transfer Case Front Shaft Speed Sensor Circuit Failure |
| P1837 | Transmission Transfer Case Rear Shaft Speed Sensor Circuit Failure |
| P1838 | Transmission Transfer Case Shift Motor Circuit Failure |
| P1839 | Transmission Transfer Case Shift Motor Open Circuit |
| P1840 | Transmission Transfer Case Shift Motor Short Circuit To Battery |
| P1841 | Transmission Transfer Case Shift Motor Short Circuit To Ground |
| P1842 | Transmission Transfer Case Differential Lock-Up Feedback Switch Circuit Failure |
| P1843 | Transmission Transfer Case Differential Lock-Up Feedback Switch Open Circuit |
| P1844 | Transmission Transfer Case Differential Lock-Up Feedback Switch Short Circuit To Battery |
| P1845 | Transmission Transfer Case Differential Lock-Up Feedback Switch Short Circuit To Ground |
| P1846 | Transmission Transfer Case Contact Plate 'A' Circuit Failure |
| P1847 | Transmission Transfer Case Contact Plate 'A' Open Circuit |
| P1848 | Transmission Transfer Case Contact Plate 'A' Short Circuit To Battery |
| P1849 | Transmission Transfer Case Contact Plate 'A' Short Circuit To Ground |
| P1850 | Transmission Transfer Case Contact Plate 'B' Circuit Failure |
| P1851 | Transmission Transfer Case Contact Plate 'B' Open Circuit |
| P1852 | Transmission Transfer Case Contact Plate 'B' Short Circuit To Battery |
| P1853 | Transmission Transfer Case Contact Plate 'B' Short Circuit To Ground |
| P1854 | Transmission Transfer Case Contact Plate 'C' Circuit Failure |
| P1855 | Transmission Transfer Case Contact Plate 'C' Open Circuit |
| P1856 | Transmission Transfer Case Contact Plate 'C' Short Circuit To Battery |
| P1857 | Transmission Transfer Case Contact Plate 'C' Short Circuit To Ground |
| P1858 | Transmission Transfer Case Contact Plate 'D' Circuit Failure |
| P1859 | Transmission Transfer Case Contact Plate 'D' Open Circuit |
| P1860 | Transmission Transfer Case Contact Plate 'D' Short Circuit To Battery |
| P1861 | Transmission Transfer Case Contact Plate 'D' Short Circuit To Ground |
| P1862 | Transmission Transfer Case Contact Plate Power Circuit Failure |
| P1863 | Transmission Transfer Case Contact Plate Power Open Circuit |
| P1864 | Transmission Transfer Case Contact Plate Power Short To Battery |
| P1865 | Transmission Transfer Case Contact Plate Power Short To Ground |
| P1866 | Transmission Transfer Case System Concern - Servicing Required |
| P1867 | Transmission Transfer Case Contact Plate General Circuit Failure |
| P1868 | Transmission Automatic 4-Wheel Drive Indicator (Lamp) Circuit Failure |
| P1869 | Transmission Automatic 4-Wheel Drive Indicator (Lamp) Circuit Short To Battery |
| P1870 | Transmission Mechanical Transfer Case 4x4 Switch Circuit Failure |
| P1871 | Transmission Mechanical Transfer Case 4x4 Switch Circuit Short To Battery |
| P1872 | Transmission Mechanical 4-Wheel Drive Axle Lock Lamp Circuit Failure |
| P1873 | Transmission Mechanical 4-Wheel Drive Axle Lock Lamp Circuit Short To Battery |
| P1874 | Transmission Automatic Hall Effect Sensor Power Circuit Failure |
| P1875 | Transmission Automatic Hall Effect Sensor Power Circuit Short To Battery |
| P1876 | Transmission Transfer Case 2-Wheel Drive Solenoid Circuit Failure |
| P1877 | Transmission Transfer Case 2-Wheel Drive Solenoid Circuit Short To Battery |
| P1878 | Transmission Transfer Case Disengaged Solenoid Circuit Failure |
| P1879 | Transmission Transfer Case Disengaged Solenoid Open Circuit |
| P1880 | Transmission Transfer Case Disengaged Solenoid Short to Battery |
| P1881 | Engine Coolant Level Switch Circuit Failure, GEM |
| P1882 | Engine Coolant Level Switch Circuit Short to Ground |
| P1883 | Engine Coolant Level Switch Circuit Failure, GEM |
| P1884 | Engine Coolant Level Lamp Circuit Short to Ground |
| P1885 | Transmission Transfer Case Disengaged Solenoid Short to Ground |
| P1886 | 4X4 Initialization Failure |
| P1890 | Transmission 4WD Mode Select Return Input Circuit Failure |
| P1891 | Transmission Transfer Case Contact Plate Ground Return Open Circuit |
| P1900 | OSS Circuit Intermittent Malfunction |
| P1901 | TSS Circuit Intermittent Malfunction |
| P1902 | Pressure Control Solenoid "B" Intermittent Short |
| P1903 | Pressure Control Solenoid "C" Short Circuit |
| P1904 | Pressure Control Solenoid "C" Open Circuit |
| P1905 | Pressure Control Solenoid "C" Intermittent Short |
| P1906 | Kickdown Pull Relay Open or Short Circuit to Ground |
| P1907 | Kickdown Hold Relay Open or Short Circuit to Ground |
| P1908 | Transmission Pressure Circuit Solenoid Open or Short to Ground |
| P1909 | Trans Temp Sensor Circuit Open or Shorted to Pwr or Gnd |
| P1910 | VFS A Pressure Output Failed Low |
| P1911 | VFS B Pressure Output Failed Low |
| P1912 | VFS C Pressure Output Failed Low |
| P1913 | Pressure Switch A Circuit Malfunction |
| P1914 | Manually Shifted Automatic (MSA) Sw Circuit Malf |
| P1915 | Reverse Switch Circuit Malfunction |
| P1916 | High Clutch Drum Speed Sensor Malfunction |
| P1917 | High Clutch Drum Speed Sensor Intermittent |
| P1918 | Transmission Range Display Circuit Malfunction |